# WLC-200 Nightly HIL Report, 2026-10-07

Bench HIL-2 · software 2.4.0-rc4 · spec ADS-WLC200-SRS v1.3 · 10 passed, 2 failed of 12 runs

Draft. The text was written with AI assistance from the fact sheet; tables and charts were made by code, and every number in the text was checked against the data.

# Summary

Night 2026-10-07, software 2.4.0-rc4, spec ADS-WLC200-SRS v1.3, bench HIL-2: 12 runs, 10 passed, 2 failed (TC-AP-03 in R03, TC-AP-04 in R04). TC-AP-03 is a new failure. TC-AP-04 has failed on all 11 nights in the history and is linked to open defect DR-1001. Several passing runs are close to their limits (see anomalies).

# Results

| Run | Test | Title | Result | Key value |
| --- | --- | --- | --- | --- |
| R01 | TC-AP-01 | Pinch detection at nominal voltage | PASS | stop latency ms = 6 ms |
| R02 | TC-AP-02 | Pinch detection at low voltage | PASS | stop latency ms = 7 ms |
| R03 | TC-AP-03 | Pinch detection at high voltage | FAIL | stop latency ms = 12 ms |
| R04 | TC-AP-04 | Pinch detection when cold | FAIL | stop latency ms = 13 ms |
| R05 | TC-AP-05 | Pinch detection when hot | PASS | stop latency ms = 5 ms |
| R06 | TC-AP-06 | Pinch protection in manual and express mode | PASS | stop latency ms = 6 ms |
| R07 | TC-MV-05 | Full close time | PASS | close time s = 3.816 s |
| R08 | TC-MV-01 | Manual movement and stop on release | PASS | stop after release close ms = 62 ms |
| R09 | TC-CAN-01 | WLC_Status timing and content | PASS | period min ms = 49 ms |
| R10 | TC-TH-03 | Opening blocked when hot | PASS | max current above 130C a = 14.76 A |
| R11 | TC-PW-03 | Motor stop at undervoltage | PASS | stop after undervoltage ms = 21 ms |
| R12 | TC-DG-01 | Hall sensor loss | PASS | dtc code = B1A01 |

# Failures

## TC-AP-03

R03 (Pinch detection at high voltage): stop_latency_ms was 12 ms against a limit of <= 10 ms on v1.3. reversal_mm passed at 163 mm. This is a NEW failure: it failed on 1 of 11 nights, and the previous 10 nights passed. History range is 4 ms to 12 ms, so tonight is the highest value on record. The run also logged a peak pinch force of 103.2 N, above 100 N (reportable per TP-AP-01). No open defect is linked.

## TC-AP-04

R04 (Pinch detection when cold): stop_latency_ms was 13 ms against a limit of <= 10 ms on v1.3. reversal_mm passed at 158 mm. This is NOT new: it has failed on 11 of 11 nights, under both v1.2 and v1.3, and the limit was <= 10 ms every night. History range is 13 ms to 15 ms. Open defect DR-1001 (S1, open): Pinch reversal late at -30 °C. The run also logged that the temperature estimate at start was -22 C while the chamber was set to -30 C.

# Anomalies on passing runs

R03 (TC-AP-03): peak pinch force 103.2 N is above 100 N, reportable per TP-AP-01.

R04 (TC-AP-04): temperature estimate at start was -22 C but the chamber was set to -30 C (soak not complete, or estimate offset). The 13 ms result may not reflect true -30 C conditions.

R05 (TC-AP-05): temperature estimate at start was 93 C but the chamber was set to 85 C (soak not complete, or estimate offset). reversal_mm = 151 mm against a min of 150 mm (margin 1 mm).

R07 (TC-MV-05): end-stop stall current of 18.2 A, lasting 42 ms. This test has no limit for it.

R10 (TC-TH-03): max_current_above_130C_a = 14.76 A against a max of 15 A (margin 0.24 A), with 2 samples within 2% of the limit. closing_possible_above_140C = 1 movement against a min of 1 (margin 0). The log starts at about 112 C and was never cooled, so 'opening possible below 120 C' is not covered by this run. Sampling was 10 ms.

R12 (TC-DG-01): stop_after_hall_loss_ms = 203 ms against a min of 200 ms (margin 3 ms). Hall loss time is only known to within one Hall interval (4 ms), so true latency is 199..203 ms and the 200 ms lower limit cannot be confirmed with certainty.

# Recommendations

TC-AP-03: open a defect for the new stop_latency_ms failure (12 ms vs <= 10 ms) and check whether it relates to the 103.2 N peak pinch force reportable per TP-AP-01. Re-run R03 on HIL-2 to see whether it repeats.

TC-AP-03: check what changed in 2.4.0-rc4 or on the bench before this run, since the previous 10 nights passed with values of 4 ms to 7 ms.

TC-AP-04: keep the failure attached to DR-1001 (S1, open) and add tonight's 13 ms result. Escalate in line with its S1 status, since it has failed on all 11 nights.

TC-AP-04: check the chamber soak and the temperature estimate offset (-22 C estimated vs -30 C set) before treating tonight's value as a true -30 C result.

TC-AP-05: check the hot-soak and temperature estimate offset (93 C vs 85 C set), and review the 1 mm reversal margin (151 mm vs min 150 mm).

TC-TH-03: review the 14.76 A peak current against the 15 A limit and the zero margin on closing movements above 140 C. Add a cooled run to cover 'opening possible below 120 C'.

TC-DG-01: decide whether the 4 ms Hall interval resolution is acceptable for the 200 ms lower limit, or capture the Hall loss time at finer resolution.

## Checks on this report

All numbers and IDs in the text match the fact sheet.