# WLC-200 Nightly HIL Report, 2026-10-06

Bench HIL-2 · software 2.4.0-rc3 · spec ADS-WLC200-SRS v1.3 · 9 passed, 3 failed of 12 runs

Draft. The text was written with AI assistance from the fact sheet; tables and charts were made by code, and every number in the text was checked against the data.

# Summary

Night 2026-10-06, software 2.4.0-rc3, spec ADS-WLC200-SRS v1.3, bench HIL-2: 12 runs, 9 passed, 3 failed. TC-AP-04 (R04) and TC-TH-03 (R10) have failed on all 10 nights in the history. TC-CAN-01 (R09) is a new failure tonight.

# Results

| Run | Test | Title | Result | Key value |
| --- | --- | --- | --- | --- |
| R01 | TC-AP-01 | Pinch detection at nominal voltage | PASS | stop latency ms = 6 ms |
| R02 | TC-AP-02 | Pinch detection at low voltage | PASS | stop latency ms = 7 ms |
| R03 | TC-AP-03 | Pinch detection at high voltage | PASS | stop latency ms = 4 ms |
| R04 | TC-AP-04 | Pinch detection when cold | FAIL | stop latency ms = 14 ms |
| R05 | TC-AP-05 | Pinch detection when hot | PASS | stop latency ms = 6 ms |
| R06 | TC-AP-06 | Pinch protection in manual and express mode | PASS | stop latency ms = 5 ms |
| R07 | TC-MV-05 | Full close time | PASS | close time s = 3.816 s |
| R08 | TC-MV-01 | Manual movement and stop on release | PASS | stop after release close ms = 62 ms |
| R09 | TC-CAN-01 | WLC_Status timing and content | FAIL | period max ms = 112 ms |
| R10 | TC-TH-03 | Opening blocked when hot | FAIL | max current above 130C a = 15.16 A |
| R11 | TC-PW-03 | Motor stop at undervoltage | PASS | stop after undervoltage ms = 21 ms |
| R12 | TC-DG-01 | Hall sensor loss | PASS | dtc code = B1A01 |

# Failures

## TC-AP-04

R04, Pinch detection when cold: stop_latency_ms was 14 ms against a limit of <= 10 ms. reversal_mm passed at 158.0 mm. This is not new: it has failed on 10 of 10 nights, on both v1.2 and v1.3, against the same <= 10 limit each night. Values over the history ran from 13.0 to 15.0 ms. Open defect DR-1001 (S1, 'Pinch reversal late at -30 °C') covers it. The run also notes the temperature estimate at start was -22 C with the chamber set to -30 C.

## TC-CAN-01

R09, WLC_Status timing and content: period_max_ms was 112 ms against the v1.3 limit of 45 to 55 ms. period_min_ms passed at 49 ms. This is a new failure tonight: it passed on the previous 9 nights, 6 on v1.2 against 90 to 110 and 3 on v1.3 against 45 to 55. The v1.3 values on 2026-10-03, 10-04 and 10-05 were 52.0, 51.0 and 52.0 ms. The log shows a 112 ms gap between t=6383 and t=6495 ms, and a frame was probably skipped. No open defect is linked.

## TC-TH-03

R10, Opening blocked when hot: two criteria failed. max_current_above_130C_a was 15.16 A against a limit of <= 15 A, with 13 samples above 15 A while the estimate was above 130 C (first at t=39110 ms at 134.0 C), described as end-stop stalls. opening_starts_above_140C was 1 against a limit of 0: opening started at t=57050 ms at 143.9 C, and the controller completed full opening travel with no inhibit. This is not new: the opening-above-140 C check has failed on 10 of 10 nights, with values from 140.8 to 143.9 C, against 'no opening above 140' each night. Open defect DR-1012 (S3, 'Opening possible above 140 °C') covers it. The history sheet does not give the current criterion's history. closing_possible_above_140C passed with 2 movements.

# Anomalies on passing runs

R05 (TC-AP-05, PASS): reversal_mm of 151 mm is within 2% of its 150 mm minimum (margin 1 mm).

R05 (TC-AP-05): temperature estimate at start was 93 C with the chamber set to 85 C (soak not complete or estimate offset).

R04 (TC-AP-04): temperature estimate at start was -22 C with the chamber set to -30 C. Same pattern as R05, in the cold direction.

R12 (TC-DG-01, PASS): stop_after_hall_loss_ms of 203 ms is close to its 200 ms minimum (margin 3 ms). Hall loss time is only known to within one Hall interval (4 ms), so true latency is 199..203 ms and the 200 ms limit cannot be confirmed with certainty.

R07 (TC-MV-05, PASS): end-stop stall current of 18.1 A, lasting 42 ms. This test has no current limit.

R10 (TC-TH-03): the log starts at about 112 C and was never cooled, so 'opening possible below 120 C' is not covered by this run. Sampling was 10 ms.

# Recommendations

TC-CAN-01 (R09): Open a defect for the 112 ms WLC_Status gap at t=6383 to 6495 ms in R09. Check the bench CAN log and HIL-2 load at that time to tell a skipped frame in the controller from a logging artefact. Then compare with R09 on the next night, since this is the first failure on v1.3.

TC-AP-04 (R04): Raise the priority of DR-1001 (S1). The test has failed on all 10 nights at 13.0 to 15.0 ms against a limit of <= 10 ms, and the limit did not change with v1.3. Also check the cold soak: the estimate at start was -22 C against the -30 C chamber setting, so confirm the test actually ran at -30 C.

TC-TH-03 (R10): Add tonight's findings to DR-1012: opening started at 143.9 C, full travel was completed, and the current peaked at 15.16 A against the 15 A limit. Consider whether the end-stop stalls above 130 C need their own defect, since none is linked.

TC-TH-03 (R10): Rerun with a cool-down so that 'opening possible below 120 C' is covered.

TC-AP-05 (R05): Check the 151 mm reversal margin against its 150 mm limit, and check the 93 C start estimate against the 85 C chamber setting. Do the same for TC-DG-01 (R12) and its 203 ms result against the 200 ms limit, with the 4 ms Hall interval in mind.

## Checks on this report

All numbers and IDs in the text match the fact sheet.